AMERICAN INTERNATIONAL

UNIVERSITY-BANGLADESH

Faculty of Science and Technology

Lab report

| Date of Submission: | 4 January 2026 |  |
| --- | --- | --- |
| Course Title: | COMPUTER NETWORK |  |  |  |
| Course Code: | COE3206 |  | Section: | I |
| Semester: | Fall | 2025-26 | Course Teacher: | Dr. Rajarshi Roy Chowdhury. |

| Group Name/No.: |  |
| --- | --- |

| No | Name | ID | Program | Signature |
| --- | --- | --- | --- | --- |
| 1 | Shahriar Adnan Ishty | 22-43377-2 | BSc [CSE] |  |

Lab-1

Experiment-1: Setting up DHCP server , WEB server , DNS server and EMAIL server in a network where there is two switches , all the servers and a pc in connected in together in a switch and one pc is connected in another switch . both switches are connected.

Description:

•  Two switches are connected directly to each other via a cross-over cable to form a single local network.

•  The network operates within a single subnet: 192.168.10.0/24.

•  Four dedicated servers are connected to the primary switch (Switch0) to provide centralized services:

DHCP Server (192.168.10.1): Automatically assigns IP addresses to client devices.

WEB Server (192.168.10.2): Hosts HTTP services.

DNS Server (192.168.10.3): Handles domain name resolution.

Email Server (192.168.10.4): Manages mail communication.

•  End-user devices (PC0 and Laptop0) are distributed across both switches.

•  Devices on the network can connect using either dynamic (DHCP) or static IP configurations.

•  This setup demonstrates the integration of multiple application-layer servers (Web, DNS, Email) and infrastructure services (DHCP) within a switched LAN environment.

Procedure:

Lab-1

Experiment-2: creating three vlan in a router with two switch and 6 pc.

Description:

- Two switches are connected to a single router to facilitate inter-VLAN routing for three distinct logical networks.

- The network is divided into three VLANs, each operating on its own subnet with a dedicated default gateway:

  - VLAN 10: Subnet 192.168.10.0, using 192.168.10.1 as its default gateway.

  - VLAN 20: Subnet 192.168.20.0, using 192.168.20.1 as its default gateway.

  - VLAN 30: Subnet 192.168.30.0, using 192.168.30.1 as its default gateway.

- The router (Router0) acts as the central gateway, providing communication between the different VLANs (Inter-VLAN routing).

- Switch0 acts as the main distribution point, connecting to the router and hosting PCs for VLAN 10 and VLAN 20.

- Switch1 is connected to Switch0 via a trunk link (dashed line), extending the network to host PCs for VLAN 30.

- A total of 6 PCs are deployed, with two devices assigned to each VLAN to test connectivity within and across subnets.

- This setup demonstrates how a Router-on-a-Stick or multi-interface router configuration allows logically separated broadcast domains (VLANs) to communicate securely in a multi-switch environment.

Procedure:

Cli script:

Switch0:

configure terminal

vlan 10

vlan 20

vlan 30

exit

interface fa0/1

switchport mode trunk

interface fa0/2

switchport mode trunk

interface range fa0/3 - 4

switchport mode access

switchport access vlan 10

interface range fa0/5 - 6

switchport mode access

switchport access vlan 20

Switch1:

configure terminal

vlan 30

exit

interface fa0/3

switchport mode trunk

interface range fa0/1 - 2

switchport mode access

switchport access vlan 30

Router:

configure terminal

interface fa0/0

no shutdown

interface fa0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface fa0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

interface fa0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

Lab-2

Experiment-1: creating four vlan in a router with three switch and 8 pc.

Description:

  Three switches are interconnected via trunk links to a single router to facilitate inter-VLAN routing for four distinct logical networks.

  The network is divided into four VLANs, each operating on its own subnet with a dedicated default gateway:

- VLAN 10: Subnet 192.168.10.0, gateway 192.168.10.1.

- VLAN 20: Subnet 192.168.20.0, gateway 192.168.20.1.

- VLAN 30: Subnet 192.168.30.0, gateway 192.168.30.1.

- VLAN 40: Subnet 192.168.40.0, gateway 192.168.40.1.

  The router (Router0) acts as the central gateway, connected to the middle switch (Switch1) to provide communication between all subnets.

  The switches (Switch0, Switch1, Switch2) use trunking to share VLAN information across the entire network.

  A total of 10 PCs are deployed across the switches, assigned to various VLANs to test traffic flow across multiple hardware segments.

  This setup demonstrates advanced Inter-VLAN routing using a "Router-on-a-Stick" configuration in a multi-switch environment.

Cli script:

Router0

configure terminal

interface fa0/0

no shutdown

interface fa0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface fa0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

interface fa0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

interface fa0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

exit

Switch1:

configure terminal

vlan 10

vlan 20

vlan 30

vlan 40

exit

interface range fa0/1 - 3

switchport mode trunk

interface range fa0/4 - 5

switchport mode access

switchport access vlan 10

interface fa0/6

switchport mode access

switchport access vlan 30

Switch0:

configure terminal

vlan 30

vlan 40

exit

interface fa0/1

switchport mode trunk

interface range fa0/2 - 3

switchport mode access

switchport access vlan 30

interface range fa0/4 - 5

switchport mode access

switchport access vlan 40

Switch 2:

configure terminal

vlan 10

vlan 20

exit

interface fa0/1

switchport mode trunk

interface range fa0/2 - 3

switchport mode access

switchport access vlan 20

interface fa0/4

switchport mode access

switchport access vlan 10

Lab-2

Experiment-2: controlling IoT devices remotely using DHCP server

Description:

  A central switch (Switch0) connects a server, a workstation (PC0), and a wireless Access Point (AccessPoint0).

  The server (Server0) performs dual roles as both a DHCP server for IP assignment and an IoT registration server for device management.

  The network operates on the 192.168.1.0/24 subnet, with the server statically assigned to 192.168.1.1.

  Four IoT devices (Appliance, Fan, Garage Door, and Light) are connected wirelessly to the Access Point.

  The Access Point acts as a bridge, allowing wireless IoT devices to communicate with the wired infrastructure.

  PC0 is used as the management console to log into the IoT server and control the devices remotely.

  This setup demonstrates how centralized DHCP and IoT services allow for the automated configuration and remote monitoring of smart home devices.

Procedure: